- mount DC-DC power supply to baseplate

- solder USB pigtail to RPI

PP5 = GND (black) // PP2 = 5V (red)

- Mount RPI with spacer to base plate

use 2.5x6+6 + 2.5x10+6 (4x)  + 4x Nylon Flat or 2.5*15+6 (4x) + 8x Nylon Flat or similar high // 2x 2.5 nuts // 2x 2.5x12+6

- Assemble and attach pihat to RPI with spacer (see above)

- connecting front plate

- connectors for backplate

1-Wire jack socket

| 1 | GND |
| --- | --- |
| 2 | 5V |
| 3 | Data |
| 4 | ------ |

5V

Data

1-Wire jack plug

| Sub-D | Farbe |  |
| --- | --- | --- |
| 1 | shield | GND |
| 2 | black | PTT, Aktiv LOW |
| 4 | red | CTCSS/SQL |
| 5 | brown | 12V OUT (GM340 Pin 10, Ignition) |
| 7 | white | Output = TX Audio |
| 9 | green | Input = RX Audio |